EV (Exosome) Project – Technical Requirements

The EV project involves consolidation and analysis of multi-instrument lab data using AI to assist in interpretation guidance. The AI system is intended to support the user in analysing results, not autonomously interpret them.

Instrument-Specific Data Handling

- Flow Cytometry (Cytoflex Nano):

  - Generates large datasets with tables and graphs.

  - AI is required to analyse possible graph configurations, suggest optimal axes, and identify particle segregation patterns.

  - File format is “.fcs” format. Generally requires fcsparcer (Python Library) to read and python code to convert the file into “.csv” format.

  - File contains metadata to take into consideration while conversion.

- Nanoparticle Tracking Analysis (Zetaview, NTA):

  - Produces size distribution data of exosomes in graphs or textual form.

  - AI to compare these results against outputs from other instruments.

  - File format is in “.txt” with parameter data.

  - File format “.fcs” also possibleTransmission Electron Microscopy

- (TEM):

  - Produces high-resolution images of samples.

  - AI must detect and mark exosomes, exclude vesicles with compromised membranes, quantify clean EVs within defined areas, and cross-validate results with Flow and NTA data.

- Western Blot:

  - Generates protein marker band images.

  - AI must adjust image contrast, read and classify active bands, and identify relevant protein markers for the user.

Data Consolidation and Insights

- Outputs from all instruments are consolidated.

- AI provides comparative insights across instruments, highlights differences, and guides the user for better data interpretation.

Customer General requirements

- The AI should guide the user through the data interpretation process

- The AI should guide the user through the lab equipment best practices (customer provided).

- The AI should take the lab equipment settings as input to fine tune the data interpretation (user Provided) and provide suggestions to improve the experiment results

- The AI should provide options to provide all the data from all the lab equipment or from some of the equipment (user provided).

- The AI should look at the “.fcs” file and the metadata from the Cytoflex machine.

- The AI may (nice to have) have options to mark the graph to separate the EV sizes, boxes to consider avg sizes.

- The AI should have options to take the EV sizes from the user for creating size averages (eg. EVs between 40-60NM is considered 40NM avg).

- The AI should convert the “.fcs” value to interpretable size of the particle base on forward and side scatter (customer provided).

- The AI should Read the “.fcs” file, understand it, and suggest graphs and anomalies based on the user input (graphs they have considered and size avgs they have considered).

  - The best choices of graphs and highest probabilities parameters will be provided by customer

  - There are 20+ parameters, the AI needs to plot them in different combinations

  - The combinations need to be returned to the AI to flag anomalies.

- The AI should read the text file provided by the NTA machine. And consider the parameters

- The AI should suggest possible corrections to the parameters used.

- The AI should use the data and compare it to the “.fcs” file and provide insight. (customer will provide the kind of insights that need to be derived).

- The UI may show the graphs visually

- The UI may have the graphs be interactive.

- The UI may keep previous chat history.

Observations During Lab Visit

- Hands-on review of instrument workflows and data outputs.

- Direct observation of Flow, NTA, TEM, and Western Blot processes to understand variability and typical data formats.

- Collected technical notes on data types, measurement parameters, and interpretation challenges for AI integration.